EXP 18 - MATLAB SIMULATION AND EVALUATION OF EYE DIAGRAM

PARAMETERS UNDER INTER-SYMBOL INTERFERENCE IN DIGITAL COMMUNICATION SYSTEMS

CODE:

clc;

clear;

close all;

%% Parameters

N = 100; % Number of bits

sps = 20; % Samples per symbol

%% Generate Random Binary Data

data = randi([0 1],1,N);

%% NRZ Encoding

tx = repelem(data,sps);

%% Time Axis

t = (0:length(tx)-1)/sps;

%% Introduce Inter-Symbol Interference (ISI)%

% Simple Low-Pass FIR Filter (Channel)%

h = [0.2 0.6 1 0.6 0.2];

% Received Signal with ISI

rx = conv(tx,h,'same');

%% Plot Results%%

figure;

subplot(4,1,1)

stairs(data,'LineWidth',1.5)

grid on

title('Original Binary Data')

xlabel('Bit Number')

ylabel('Bit')

ylim([-0.2 1.2])

subplot(4,1,2)

plot(t,tx,'LineWidth',1.5)

grid on

title('Transmitted NRZ Signal')

xlabel('Time')

ylabel('Amplitude')

ylim([-0.5 1.5])

subplot(4,1,3)

plot(t,rx,'r','LineWidth',1.2)

grid on

title('Received Signal with Inter-Symbol Interference (ISI)')

xlabel('Time')

ylabel('Amplitude')

subplot(4,1,4)

eyediagram(rx,2*sps)

title('Eye Diagram under ISI')

OUTPUT: